Intelligent Component Insertion Machine

Operation Manual

1 CLICK SMT TECHNOLOGY CO.,LTD

Address: Room 204 & 206, Building A, NO.2 Factory zone, Science & Technology Industry zone, Dalingshan Town, Dongguan City, Guangdong Province, China. Post code: 523825

Contact Person : Wu Guoguang Tel：+8613924639874

Contents

1Description of Equipment	4

2Introduction to the Main Function Interfaces of the Software	7

2.1The Interface of Starting up	7

2.2Main Interface of the Software	8

2.3Manual Interface	10

2.4Interface of Settings	12

2.4.1Interface of Product Management	12

2.4.2Interface of Production Settings	13

2.4.3Interface of System Settings	18

2.4.4 Process Settings	33

2.4.5User Settings Interface	34

2.5Program Selection Interface	37

2.6Statistic Interface	37

2.6.1Production Data	38

2.6.2Alarm data	39

2.6.3Operation Log	40

2.7Introduction to Other Interface	42

2.7.1CAD Data Selection Interface	42

2.7.2Visual File Selection Interface	43

2.7.3Interface of Route Settings	44

3Settings Process of New Product	46

3.1Flow Diagram	46

3.2Create New Models	48

4Edit Production Progress	52

4.1PCB Data	52

4.2 PCB Configuration	53

4.3 Conveying Settings	57

4.4Mark Settings	60

4.4.1 Mark Interface	61

4.4.2Operational Process of Mark Points	65

4.5 Insertion Data	69

4.6Use / Non-use of Production components	80

4.7 Sequence Settings	81

4.8Trial Insertion	83

4.9Other Functions	85

4.9.1	85

4.9.2 Automatic Nine-point Calibration	85

5 Description of Visual System	86

5.1 Overview	86

5.1.1 Main interface	86

Tool bar	87

Menu bar	87

5.1.2 File production interface	88

File selection and deletion	99

5.2 Processing tools	101

5.2.1 Shape model match	101

5.2.2Right angle positioning	104

5.2.3Gray model match	105

5.2.4 Blob positioning	106

5.2.5 fit circle positioning	107

5.2.6 fit rectangle positioning	109

5.3 Fast model	110

6 Starting Process	113

6.1  Check before Operation	114

6.2 Starting Steps	115

7  Finish Production and Turn off Power	118

1Description of Equipment

1.1Introduction to Equipment：

Intelligent component insertion machine is a piece of equipment used for inserting components into a solder printed circuit PCB or into an electric circuit PCB to which the adhesives are applied. The component is firstly clamped from the feeder, and then the camera will recognize the center of the component. Next, according to the CAD coordinate data of the component in the PCB, the position coordinate of the component can be derived, and finally the equipment will insert the component in the designated position in accordance with the program.

Intelligent component insertion machine adopts EPSON six-axis or four-axis robots as the main working mechanism of the equipment, which makes the whole equipment look simple in structure, but faster and more stable.

1.2Structure of Equipment

status indicator

Air pressure indication

PCB Conveyor belt

head of insertion

back operating button

console

Recycling bin for defective products

vision correction system

feeder input connector

air switch

Power switch

1.3Size of Equipment：

1.4Easy Starting up (Note: Before starting the equipment, please confirm that the robots and the working head are in the non-interference position.)

Start procedure :

- Turn on the breaker .

- Press power button.

- Release the emergency stop switch .

- Double click software icon on the desktop and wait for the software initialized.

- Click home , and home button would light on the control panel.

Before operating the machine, the user must completely understand the basic structure and operation methods of the interface of software system.

2Introduction to the Main Function Interfaces of the Software

2.1The Interface of Starting up

■demonstration of start-up interface

Fig. 2-1-1

As shown in Figure 2-1-1, it is the start-up interface of the equipment software. And at the time of starting up, the operating parameters of entire equipment will be processed. The progress bar at the bottom of the start-up interface indicates the current start-up progress, and the text below the progress bar indicates the current start-up status of the equipment. In this way, the user will have a clear idea of whether the equipment is started properly.

2.2Main Interface of the Software

■demonstration of main interface

6

5

4

3

2

1

Fig. 2-2-1

■instruction of specific functions in the interface：

- navigation bar：

The navigation bar can be clicked to switch to different major interfaces, including mainly production interface (main interface), setting interface, model selection interface, counting interface and manual interface.

- production indicator：

This section shows the model of the product being produced currently, production status, efficiency of the equipment, and the operating status of the equipment. In case of an alarm, the green light will turn red, and with double click on the red light, there will be instructions of the alarm contents and its cause and corresponding solutions displayed on the interface.

- control button：

The control buttons include Bypass, automatic production (pause), home, and cycle stop.

Bypass: Click the button , and the equipment will be in the status of straight passage. And at this time, the feed belt of equipment is working, and the entire equipment is used as a conveyor line. (Generally, when the equipment fails and the failure cannot be solved in a short time, the equipment will be in the status of straight passage so as not to affect the operation of the entire production line.)

Automatic Production (Pause): The same button is qualified with two functions of Automatic Production and Pause. When the button of Automatic Production is clicked, the name of button will appear to be “Pause”. At this time, the equipment will automatically operate the product production. And when the button is pressed again, the equipment will pause, and the name of button will appear to be “Automatic Production”.

Home: The entire machine and each part will reset.

Cycle Stop: During the automatic production process, after the button is pressed, the equipment will enter the state of cycle stop, and the equipment will stop running after the current production on the belt is completed.

- User information：

The name of the currently logged-in user is shown here.

- Log in：

Click on the button and enter the user login interface. It is used for users to log in so as to obtain the permission to use the equipment.

- Exit：

Exit the software.

2.3Manual Interface

The functions of buttons in the manual interface are described as follows.

Figure 2-6-1

The manual interface is shown in Figure 2-6-1. This interface is mainly used to activate the various parts of the equipment and to confirm whether they are operating normally.

------------------------------------------------------------------------------

■Conveyor 1

[Convey] button

Rotate the inlet station to activate the feed belt.

[Require output ]

Realize the mandatory signal output to the upper machine for permission of PCB entry.

---------------------------------------------------------------------------------

■Conveyor 2

[Forward ] button

Perform the forward rotation (in the direction of PCB outflow) of the feed belt in the intermediate station of the insertion machine.

[Reverse] button

Perform the backward rotation (in the direction of entrance) of the feed belt in the intermediate station of the insertion machine.

---------------------------------------------------------------------------------

■Conveyor 3

[Forward] button

Realize the rotation of feed belt in the entrance station.

---------------------------------------------------------------------------------

■Positioning motor

[Clamping on] button

Clamping motor move to adjust the positioning for tightly clamping the PCBs.

[Stopping] button

Stopping motor move to stop the PCBs.

[Clamping off] button

Clamping motor move to adjust the positioning for release of PCBs.

[Home] button

Stopping and clamping motor return to the origin, and the position scale will become zero after returning to the origin.

---------------------------------------------------------------------------------

[All ready for one cycle] button

According to the determined PCB width, the track will become narrower to the set width.

[Load PCB]button

Convey 2 load PCB.

[Manual-Unload PCB]button

Conbey 3 unload PCB.

Activate the simple tray counter .

Do not operate during automatic production operation.

[Robot jogging ]

Robot jogging moving

[Robot power off ]

Power off robot

[Gripper reset when error  ]

Reset gripper when error appears.

[Robot I/O ]

Display the Robot I/O

[Gripper test  ]

Select gripper and set open/close time and manually test it .

2.4Interface of Settings

Figure 2-3-1

The settings interface is shown in Figure 2-3-1. The settings interface is divided into four parts including product management, production settings, system settings, and user settings.

2.4.1Interface of Product Management

Figure 2-3-2

The product management interface is shown in Figure 2-3-2. This interface is used to create new product models and display the components produced in terms of different models of product.

1. Current program: Displays the current program of product managed by the equipment.

2. New: New product model.

3. Copy: Copy the data of the selected product model to the new product model.

4. Delete: Delete the selected model.

5. Edit: Edit and select the components requiring to be produced according to the selected models.

6. Load file : Load the file .

7. Next step: Enter the data setting interface of PCBs production. (Double click on the component display will also lead to the entry.)

2.4.2Interface of Production Settings

- Data of PCB

4

3

1

2

Figure 2-3-3

As shown in Figure 2-3-3, the production settings interface is divided into three parts including PCBs data, Mark settings, and insertion data.

1. Production setting interface navigation bar.

2. PCB Size setting and other PCBs data.

3. Conveying setting.

4. Save or restore the data.

After the data setting is completed, the user needs to click the button of “Save” to write the data to the motor driver, and then click the “Rotate”, “Move” and other buttons to test whether the speed or position is proper. After modifying the data, the user is required to click the "Save" button, or the user can click the "Restore" button to restore the data to the original state before this modification.

2. Mark Settings

2

1

Figure 2-3-4

The Mark settings interface is shown in Figure 2-3-4. The interface is divided into two parts include robot activation and data display setting.

1. Robot activation: Set the displacement by selecting the step distance or manually inputting the displacement or moving the slider, and click the button of stepping motion so as to allow the robots to perform the corresponding motion.

Continuous movement: When the function of continuous movement is selected, the mouse requires to be clicking on any direction button with continuous pressing the right button, and the robot will continue to move in this direction until the user stops pressing the right mouse button or the alarm goes off due to the robot outside the specified area.

Mark1, Mark2, Mark transition point: Select the settings of current interface as the data settings of a certain transition point.

2. Data display setting: This part will display the coordinate values of visual processing pixel, the coordinate values of currently running robots, and the coordinate value of robots under the current point setting.

3. Move: Move the robot to the set coordinates.

4. Save: Save the CAD data, visual file and set position of the robots under the current point setting.

5. Next step: Enter the “Insertion Data” interface.

3.Insertion Data

3

1

2

Figure 2-3-5

The insertion data is shown in the Figure 2-3-5.

1,  component selection setting: set the production components under the current model .

Line setting: Sequence the picking by clamping grippers, photographing, and insertion.

2.  Set the selected components：

ID : Click the button of “Select” to open the CAD data interface and select the corresponding component ID.

No. Input the component No.

Name: input the component name.

Component mark :Select use mark or not .

Feeder Port: Select the communication interface to which the components feeder is connected from the drop-down list. Click the “Details” button to open the feeder connector test interface, where the user can select the number of the connector to test whether the it is exactly connected to the current feeder.

Gripper port: Select the number of clamping gripper for this components from the drop-down list.

Gripper code : Double click on the display frame and select the corresponding number of clamping gripper for this components from the pop-up list.

Picking point: Open the “components Picking Point” setting interface and set the components picking position.

Vision Settings: Open the “Visual Settings” interface and set the visual rotation center and pixel ratio of the component and other data.

Blind insertion: Set the component insertion without visual correction.

Blind insertion after photographing failure: In case of the failure in visual process, the components is set not to be discharged and the equipment will perform blind insertion.

Photo position: Open the “Photo position” setting interface and set the component photographing location.

Discharging transition point: Open the “Discharging Transition Point” setting interface and set the position of the components discharging transition point. (This point can only be set in use.)

Re-insert: Select whether to perform supplementary inserting in case of NG components insertion.

Immediate Re-insert: Select and set the timing of supplementary inserting in case of NG components insertion, including immediate supplementary inserting or supplementary inserting after the cycle is completed. (This function can be set only after the function of supplementary inserting is selected)

Insert angle: Set the angle between the position of the component inserting and the state of components at the components picking spot.

Trial : Opening the “Trial Insertion” interface, the user can start the currently selected components for one trial insertion. During this process, the user can adjust the visual data or compensation settings of insertion point.

Insertion position: Open the "Inserting Position" setting interface, and set the component insertion point and insertion transition point. (it is generally set by trial insertion function)

3.    Data recovery and storage: Restore and save the setting data of the selected component.

Copy: Copy the data of other components to the currently selected component.

2.4.3Interface of System Settings

2

3

1

Figure 2-3-6

The system settings interface is shown in Figure 2-3-6. The system settings are mainly used to determine the basic operation data of the equipment, including track settings, robot settings, visual settings, clamping gripper settings, IO settings, and process settings.

1.  Navigation bar of the system settings

2.  Sub-navigation bar of the system settings

3.  Settings content

- Conveyor set

  - The Mode of Three-Section conveyor

The settings interface of three-section line mode is shown in Figure 2-3-6. It is mainly used to set the slow speed, acceleration and deceleration of speed and duration of slow running of each belt line.

Conveying width

Figure 2-3-7

The conveying width interface is shown in Figure 2-3-7. Set the motor back to the origin. Define the conveying width by clicking on the button of “Close” or setting the movement test of “PCB Width Test”, and save the setting data.

  - Clamping Motor

Figure 2-3-8

The clamping motor setting interface is shown in Figure 2-3-8. Set the motor back to the origin. Define the height for blocking conveying by clicking on the button of “Up” or setting the movement test of “Height for Blocking conveying”; in order to realize the positioning of products, the “Spacing Positioning” is equal to the combination of “Current Position” with “PCB Thickness”, and save the setting data after defining the position.

  - Cycle Adjusting

Figure 2-3-9

Figure 2-3-9 shows the track cycle debugging interface. This interface allows for periodic or single-step test of each belt line and shows the running status of each belt line.

Single step mode: One Click for one stepping forward of the belt line.

Periodic mode: The periodic running of belt line is required to wait in case that the signal fails to meet the requirements.

Reset: Reset the belt line.

  - Communication

Figure 2-3-10

The settings interface of track motor communication is shown in Figure 2-3-10. Select and set the COM port and baud rate for motor communication from the drop-down list.

- Robot set

Figure 2-3-11

As shown in Figure 2-3-11, the robot settings interface

  - Hand

The robot hand settings interface is shown in Figure 2-3-11. The user can read tools point for robot hand .

  - Nine-Point Calibration

Figure 2-3-13

The settings interface of automatic nine-point calibration of robots is shown in Figure 2-3-13. Use the “Adjust” button to open the settings interface of “Visual Reference Point” and “Posture of Clamping gripper” to set the corresponding position of the robot. After the setting is completed, click the “Starting Nine-Point Calibration” button to automatically perform the nine-point calibration.

  - Conveyor calibration:

Calibrate the coordinate of conveyor , the calibration process is completed before delivery .

  - Calibration center

Figure 2-3-14

Calibrate the bottom CCD position with a calibrating pin .

  - Function setting

Figure 2-3-15

Figure 2-3-15 shows the Mark starting point settings interface. This interface is used to set the robot position of the Mark starting point.

Speed :Set the speed of robot

Home :Set Robot home position.

Discard position :Set component discarding position.

Machine set : Set start point , fiducial mark point .

Pressure detect : Detect the air pressure .

Custom feeder : Set custom feeder.

  - Cycle Adjust

Figure 2-3-16

Figure 2-3-16 shows the robot cycle debugging interface.

1. Low power & high power: Set the operating power of the robot.

2. Stop position : Move the positioning motor to the position for blocking conveying.

3. PCB stopping: Move the positioning motor to the determined position.

4. Test repeatbility of gripper: The robot will perform the single-step test.

5. Cycle mode: The robot will perform the periodic running.

6. Reset: The robot is reset.

7.Manual signal : The signal will be sent to the robot, meaning the product has been successfully positioned.

Conveying finished :send signal of conveying finished to robot

One cycle ready: send one cycle ready signal to robot .

Disable feeder signal: Shield the communication signal between the feeder and the machine.

- Vision set

Figure 2-3-17

The vision settings interface is shown in Figure 2-3-17. Set the brightness of visual illumination and visual file template.

- Gripper set

Figure 2-3-18

The gripper set interface is shown in Figure 2-3-18. Set the number of grippers. Click on the “Clamping grippers Information Pool”, and enter the clamping grippers information settings interface.

Figure 2-3-19

The settings interface  information is shown in Figure 2-3-19.

1. Code: The number of clamping grippers is set according to the field requirements, and there must be no repetition.

2. Name: The name of the grippercomponent should correspond to the component name in the insertion data as much as possible for easy searching.

3. Total length: The total length of the grippers when holding the component.

4. Height: The extra length of components exceeding the the bottom of the gripper

5. Pick in advance :picking in advance time setting

6. Grip delay: The delay time at the same time of clamping the gripper cylinder.

7. Insert in advance :insert in advance time setting

8. Insert in place time: Set inserting in place delay time.

9. Release time: The delay time at the same time of opening the release cylinder.

10.Picking speed :set picking speed.

11.Inserting speed :set inserting speed.

12.Inserting detect :select inserting detecting function or not

13. Type: Select the type of clamping gripper, including clamping grippers or suckers.

14. Add: Add the item of clamping grippers information.

15. Modify: Modify the information data of clamping grippers.

16. Delete: Delete the item of clamping grippers information.

In case of necessary modification, click the line to be modified in the selection list, modify the corresponding item at the top of the list, and then click “Modify”, and finally the data in the list will be successfully modified. After the modification is completed, the user needs to return to the interface shown in Figure 2-3-18 to perform the data storage. If the model of the modified clamping grippers data fails to be modified correspondingly, please write the modification into insertion data.

- Light tower Settings

Click the Settings → System Settings → Tri-color Lamp Settings, and there will be the following window.

Figure 2-3-19

The light tower settings interface is shown in Figure 2-3-19.

Flash default: One flash per second.

When the selection of running status flashes, please determine the time of flashing on/off. When the setting is completed, click the button of [Save] to make sure the effective setting. Please set various running statuses according to the field conditions.

2.4.4 Process Settings

Figure 2-3-20

The process settings interface is shown in Figure 2-3-20.

In case of the Mark point failure, the process setting interface can be used to set the processing method, the number of supplementary inserting and photographing, and the time of production shifts.

2.4.5User Settings Interface

The user settings interface is used to set user permissions, add or delete users, and modify user passwords. The functions include password change, function management, user management, and post management.

1、Password Change

Figure 2-3-21

Figure 2-3-21 shows the password change interface. Enter the user name and the original password, set a new password, and the successful password change can be realized.

2、Function Management

Figure 2-3-22

Figure 2-3-22 shows the function management interface. The function management interface is used to set the visibility of the system and the effectiveness of the function.

3、 User Management

Figure 2-3-23

User management interface is shown in Figure 2-3-23. it is used to set the user level and other information.

4、Post Management

Figure 2-3-24

The post management interface is shown in Figure 2-3-24. It is used for high-level users to modify the post permissions of low-level users.

2.5Program Selection Interface

Figure 2-4-1

The program selection interface is shown in Figure 2-4-1. When selecting the production program on shift, the user needs to set the planned output on shift, the planned production time, and the planned product cycle, then click the "OK" button to select the model and to write the model data. The “Trial Operation” and “Null Operation” functions are the same as that of “Product Management” in “Settings”.

2.6Statistic Interface

The statistics interface is shown in Figure 2-5-1. It is used to realize the statistics of production data, alarm content and records, production operation records, and equipment operating status information.

1, Statistics interface navigation bar

2. Statistical content

2

1

Figure 2-5-1

2.6.1Production Data

The production data statistics interface is shown in Figure 2-5-1.

component Details: Click the “component Details” button to open the statistics of each component required for the currently produced product.

2.6.2Alarm data

Figure 2-5-2

The alarm content interface is shown in the Figure 2-5-2. It is used to add or delete the information of the alarm contents. Moreover, the operational guideline interface of alarm may be opened by the "Guideline" button (shown in Figure 2-5-3), and according to the alarm number, the user can inquire the alarm contents, cause and processing methods, and can also modify the alarm content guidelines in such an interface.

图2-5-3  Figure 2-5-3

2.6.3Operation Log

Figure 2-5-4

Figure 2-5-5

The operation log interface is shown in Figure 2-5-4, while the alarm log is shown in Figure 2-5-5. The operator's operation record during a certain period can be inquired according to the operator, the operation start time and the finish time, and the alarm record during the period can be inquired according to the alarm number, the operator, the record start time and the end time.

2.7Introduction to Other Interface

In addition to the above main interfaces, the equipment software also needs to employ some auxiliary interfaces. These auxiliary interfaces will be briefly introduced as follows.

2.7.1CAD Data Selection Interface

Figure 2-7-1

The CAD data selection interface is shown in Figure 2-7-1. When the CAD data of each component or Mark point on the PCB is required, double-click on the display frame will open the CAD data selection interface. The selection list can display the data related to coordinates and component ID of all components and Mark points on the PCB of the model of currently-produced product (the component ID in the insertion data is also selected from the list). This list is qualified with the function of filtering, facilitating quick data finding .

After selecting the required data, double-click on the data line can read the data out to the display frame of the upper interface.

2.7.2Visual File Selection Interface

Figure 2-7-2

Figure 2-7-2 shows the visual file selection interface. Visual processing requires the setting of a visual file. With double-click on the visual file display frame, a visual file list will pop up. Select the desired file in this list and perform a double-click to display the selected file data in the upper interface.

2.7.3Interface of Route Settings

Figure 2-7-3

Figure 2-7-3 shows the line settings interface. Click the “Line Settings” button on the “Insertion Data” interface to open the line settings interface. This interface is used to set the use of the grippers and also to sequence the picking, photographing and insertion. Check the clamping grippers number on the left, and the corresponding clamping grippers number will be displayed in the three lists on the right. Select a clamping grippers number in the corresponding order list, and move its position in the list by clicking the up arrow and down arrow. This method can achieve the function of sequencing. After the sequencing is completed, click the "Save" button to save the determined order.

3Settings Process of New Product

The production process of new products will be introduced as follows.

This chapter will describe the Step 4, Step 5, Step 6 and Step 7 (“Automatic calibration”) of the production of new products.

3.1Flow Diagram

Step1 check machine before operation

Check the conditions around the robot and perform daily inspections.

Step2Power on

Make sure the main air pressure above the 0.5 Mpa.

Step3Set the machine back to the origin

Check if there is any foreign matter inside the equipment before operation.

Step4CreateNew Program

Create the same name and program as the production PCB (for easy management of models and easy switch of models).

Step5Create the grippers data base

Step6Edit production program

PCB data

Mark point data

Visual data

Insertion Data

Step7Automatic Calibration

The component photographing height will be automatically calibrated, and the automatic calibration result will then be saved (in case of abnormal insertion such as positional offset and centering failure, inspect the the visual mode data and perform the automatic re-calibration).

Step8Production

Step9Finish the production

Step10Power off

3.2Create New Models

The operational method of Creating new program will be described as follows.

- Click the button of [Settings] to open the interface of settings, and click the button of [Product Management].

- Press the [New] button, and the new model dialog frame will be displayed, and finally, click the [Yes] button.

- Enter the name of newly-added product and click the button of [Yes]

Enter the letters, numbers, and symbols no more than 60 characters.

The setting of name is displayed in the production process as well as in production, so it is recommended to use a simple and well-understood name.

。

Enter the newly-added product name. No repeating, blank and omission.

4.Check the format of data source.

There are two kind of data as follows.

(1). [In DXF format]

AutoCAD type files (the software of AutoCAD 2010 supports the conversion into DXF format), graphic data.

(2). [In CSV format]

Excel data type, table data.

。

At present, the insertion machine only supports DXF format and CSV format (data content supports the actual measurement, drawing and production, and the conversion into DXF or CSV format).

4Edit Production Progress

The production progress consists of 4 projects.

The production process is realized in the order of the PCB data - Mark settings - visual data - insertion data.

| Type of Data | Content |
| --- | --- |
| PCB Data | Process the overall dimensions of the PCB |
| Mark Settings | Process the data on PCB offset position and rotation angle |
| Visual Data | Process the data required for component height and centering coordinates |
| Insertion Data | Process the reclaiming points, photographing points, coordinates of inserting points, component names, etc. |

4.1PCB Data

The new product must be set as follows in terms of the PCB data.

The user must click the [Save] button to make the settings or modification of the parameters effective.

4.2 PCB Configuration

PCB Configuration

◆Simple PCB: A simple PCB refers to a PCB on which there is only one circuit PCB.

◆Matrix circuit PCB: It is a multi-circuit PCB, where there are a plurality of identical circuit PCBs.

The data of a circuit (this circuit is called "reference circuit") is created in the insertion data, with the circuit configuration information (number of circuit rows, number of columns, line spacing, column spacing, etc.) in the PCB data.

(1).Overall Dimensions of simple PCB

Enter the PCB overall dimensions.

If the model PCB (discarded part) is included, enter the overall size including the model PCB.

If the PCB used as return carrier (discarded part) is included, enter the width of the carrier.

Length

Width

If the PCB size is wrongly input, given to the existing track width, there may be slack conveying or the falling down of PCB, thereby damaging the PCB.

(2).PCB thickness

Enter the PCB thickness.

This value is used to determine the rise height of the support table when the PCB is centered.

Thickness

Borad

Thickness

Thickness

If an incorrect value is entered, it may cause the excessive pressure of support table on the PCB, leading to the deformation or damage to the PCB. Or the support table fails to tightly clamp the the PCB, leading to deviation during the insertion and further the poor insertion.

(3). Design Deviation

Enter the distance from the PCB position reference (origin) to the PCB design endpoint.

When using the CAD data, if the user wants to use the predetermined origin (the CAD origin or the origin independently owned by the company) as the PCB position reference (origin), input the distance from the PCB position reference (origin) to the PCB design endpoint specified by CAD or the like.

PCB endpoint

PCB origin

Borad

When the convey is the front reference and the PCB moves from the left to the right, the PCB design endpoint is the "reference position".

When the "PCB position reference" (origin) is in the lower left, enter the value of (Y1, 0) respectively as the PCB design offset coordinates of X and Y, and Y1 should be a positive value.

(4).Dimensions of PCB (Matrix PCB)

A matrix circuit PCB refers to a PCB in which all circuits have the same angle, with identical sizes of the circuits and spacing between the circuits.

Outline dimension Length

Enter the overall dimensions of the circuit (including all insertion coordinates and dimensions marked within the circuit)

Small PCB

Width

Outline dimension

Width

Small PCB

length

By default, the reference circuit of the matrix PCB tends to perform the insertion initially from the circuit nearby the pin along the fixed boundary the of track.

(5).Number of circuits (matrix circuit PCB)

The transfer is defined in the direction of a row, and the direction perpendicular to the conveying is defined as a column, and the user is required to enter the number of circuits in each direction.

(6). Spacing between PCB (matrix PCB)

The spacing in the conveying direction is defined as the line spacing, while the spacing in the direction perpendicular to the conveying is defined as the column spacing. The user is required to enter the spacing between the circuits in each direction (when a negative value is input, the insertion direction is reversed)

(7).Photographing settings of Mark point (matrix  PCB)

◆	Photographing of Mark point on the PCB: in terms of whether a simple PCB or a matrix circuit PCB (a matrix circuit PCB is integrate without disconnection), the photographing of any two Mark points on the PCB is required.

◆Photographing of Mark point on the matrix circuit PCB: in terms of a matrix circuit PCB, the repeated photographing of Mark points of every circuit on the matrix circuit PCB is required for centering.

Matrix PCB

Vehicle

Check the settings according to the current PCB type to avoid accidental collision.

4.3 Conveying Settings

4.3.1 Conveying settings parameters

The conveying mechanism is divided into three sections, with each section of belt running independently.

Station 1: When the substrate flows from the left to the right, the conveying belt is defined as station 1, qualified with the buffer function for PCB entry.

Station speed: the speed of conveying substrate ranges from 100 to 2500

Station 2: Intermediate belt, defined as station 2, with the role as the insertion station.

Station speed: the speed of conveying substrate ranges from 100 to 2500

Positioning speed: the running speed of support table ranges from 2 to15 (rps/s, the number of revolutions per second)

Station 3: When the substrate flows from the left to the right, the section is used to transfer the PCB on which the insertion is completed and is defined as station 3, with the buffer function for PCB exiting.

Station speed: the speed of conveyingPCB ranges from 100 to 2500

The speed of the station should not be too high or too low. Low speed will affect the overall period, and the fast insertion of components on the PCB will lead to the falling or deviation of the product at the time of start and stop, resulting in the bad flow into the next process.

4.3.2  Width Adjustment of Conveyor

During the production process, whenever the production model is switched, the conveyor will automatically adjust the width according to the PCB size of the model.

Width of rail adjustment

Click the [Reset] button, and the sensor mounted on the track will detect the PCB, and then the equipment will terminate the reset.

When the conveyor is automatically adjusted, if there is a PCB on the track, it will fall off or be damaged (there are detection dead zones for the sensor mounted on the track , and not all the positions of the track can be detected). Before performing the reset when switching the production model, all the PCBs on the track need to be manually removed.

4.4Mark Settings

The Mark settings must be in accordance with the instructions on the operation interface. The basic operation flow is shown in the figure below.

click the button of [Save]

click the button of [Finish the Demonstration]

The center line of camera should be aligned with the Mark points

Click the direction buttons and move to the desired direction

real-time photographing

select and design Mark points

select and mark the specific Mark points

tightly clamp the PCB

put the PCB in place

4.4.1 Mark Interface

The Mark settings interface is introduced as follows.

9

5

6

8

7

4

3

2

1

◆Compensate for the difference between the design size and the actual size and the error of direction of rotation.

When there are multiple marks on the PCB, within the scope of the insertion, two points should be selected on the diagonal line (when the distance is farther, and the error of rotation direction will be smaller).

|  | Button Name | Function |
| --- | --- | --- |
| 1 | Mark points | Marked points on the diagonal lines of PCB, defined as Mark1 and Mark2. |
| 2 | Direction | Use this button when you want to move to the desired direction. |
| 3 | Stepping mode | Use the direction buttons to produce the specified displacement during the stepping motion. |
| 4 | Camera identification | Identify points within the field of view of camera. |
| 5 | Visual file | Select the mark point visual file completed in advance. |
| 6 | CAD data | In case of no CAD data, select the mode of no data. |
| 7 | CAD point selection | Select the design mark points. |
| 8 | Finish the demonstration | The current photographing position is the actual mark point. |
| 9 | Save | Save all data on the current interface. |

When the design mark points are inconsistent with the actual mark points, the calculation of the compensation value for PCB offset is not accurate. Phenomenon: When the PCBs stop at different positions of the insertion station (within the camera's field of view), all insertion by the robot fail.

In order to avoid personal injury, please do not put your hands inside the equipment during teaching, or draw your face and head close to the equipment.

◆Visual File：

Double click with the mouse left button

Use the left mouse button to make the double click on the name of visual file. After the visual file selection interface is displayed, select the the mark point visual file completed in advance. and make a double-click to confirm the selection.

In terms of Mark1 and Mark2, the user must choose to design mark points and visual files.

4.4.2Operational Process of Mark Points

Set the Mark Points following the steps below：

Step1Put the PCB on the conveying track.

Step2Lift the support table to fix the PCB (manual interface [Move the Positioning] button)

Check if the PCB is evenly supported and tightly clamped. Gently shake the PCB to confirm there is no displacement of the PCB, and gently press the PCB to confirm there is no warping PCB. If the PCB is not warped, and has been uniformly supported, the PCB is considered to be normal.

Step3Select the Mark point to be set.

Step4Select and design mark points

◆design mark points：

Click the button of [Select CAD Point] to open the interface of select CAD point. Press the left mouse button to select the design timeline and frame mark point, and click the [OK] button.

When the CAD data file is opened, if the displayed graphs are too small or the drawings fail to be displayed, the user can change the graphic dimensions to be in millimeters.

Step5Continuous Photographing

Click the continuous acquisition button

Click the button of [Continuous Photographing] and the camera will display the current position image in real time.

In case of blank or invalid visual files, the continuous photographing cannot be realized.

Step6Click the direction buttons to move the camera to the real-time photographing positions.

Step7Click the direction buttons to move the camera to the real-time photographing positions. And make the center line of camera in alignment with the mark point on PCB (the mark point on PCB can be perforated or pinned).

Step8Click the button of [Finish the Demonstration].

Step9Click the button of [Save], write the data into robot and make the settings effective.

4.5Insert Data

New component must be set as follows in the insertion data.

Click the button of [Save] to make the settings or modification of parameters effective.

4.5 Insertion Data

The components data is described as follows：

The creation of components data should be realized according to the following steps

Step1component ID

Click the button of [Select] to open the interface of select CAD point. Press the left mouse button to select the design timeline and frame mark point, and click the button of [OK].

Step2component No.

Annotations for components are used to make it easier to distinguish the similar components from each other. It has no direct impact on inserting.

There are no more than 16 characters in terms of “component Number”.

The user can enter uppercase characters and lowercase characters, special symbols, and so on.

The annotation can be omitted.

Step3 Name of component

Display the component name. (Display the name used in the insertion data)

Step4Feederport

Specify the connector for the component feeder.

There are three reclaiming positions for each component, so there are three reclaiming connectors, including one main feeder connector and two spare feeder connectors.

Click the button of [Select], and the interface of feeder port will pop up.

When the time of alarm of no component is set to be 0, the alarm is considered to be go off three seconds later by default, and the duration of automatic production without component will be recorded immediately.

Step5Gripper position

Specify the axis number used for inserting.

Select the axis number from the pop-up menu.

Step6Gripper code

Select the grippers code for the corresponding components.

Make double click to select the data in the surface of clamping grippers. See the the settings of clamping grippers pool in the Chapter two.

Step7picking Points

Set the coordinate position of the component feeder .

There are three reclaiming positions for each component, including one main feeder position and two spare feeder positions. (When the spare feeder is initiated, the component must be are the same mode and also same type of pin)

When the three feeders are activated at the same time, give the priority to the component picking from the prepared feeder.

The data required for inserting must not be omitted. The reclaiming position should be accurate to avoid accidental collision and damage to the robot.

Step8Vision Settings

◆Before the visual settings, the user should be sure to confirm the centering posture of component photographing after finishing the photographing point demonstration.

Current calibration value

Calibration axis number

Image display

Automatic calibration start

Click the button of [Visual Settings] button, and the interface of visual settings will pop up. And follow the steps below.

select the components

Move to photographing point

preparation before automatic calibration

NO

Finish the calibration

Click the button of [start Automatic Calibration]

Enter the amount of calibration movement

Make calibration file

YES

Select the calibration centering file

Input XY movement amount: at the time of automatic calibration, the values of X-axis and Y-axis movement distance can not be less than 3MM.

Function options：

Blind insertion: set the component insertion without visual correction (during automatic insertion production cycle, the photographing of component will not be performed).

Blind insertion after photographing failure: In case of the failure in visual process, the components is set not to be discharged and the equipment will perform blind insertion.

Starting the automatic calibration, the photographing of robot X-axis and Y-axis movement calibration is used to calculate the calibration value. The entered X-axis and Y-axis calibration movement cannot be less than 3MM.

Choose to use the same axis number can avoid automatic calibration. In case of choosing to use different axis number or insignificant adjustment of the photographing centering posture, it is necessary to restart automatic calibration.

Step9 Photo Position

Set the photographing centering posture of components.

Direction button

Click the button of [Move to Default Photographing Point], and the robot performs the movement, stops at the top of the reference photographing posture of the system, and then the user needs to click the direction buttons to make slight adjustment, and finally click the button of [Save].

Tips: Click the right mouse button will lead to the movement to position below [Default Photographing Point].

The data required for inserting must not be omitted. The maximum component size should not exceed 50MM length * 50MM width, and the component should be placed in the fixed camera's field of view and the clear and visible image should be ensured. It is preferable to place the component in the center of the camera's field of view.

Step10component Discharging Transition Position

It will be used in case of bypassing when there is an interference object on the discharging path. Set an avoidance point during finishing the path.

Check the button of use first.

Then click the [component Discharging Transition Position] button to open the NG transition point setting interface for demonstration. Click the direction button to move to the desired position and click the [Save] button.

6

1

6

5

4

3

4

2

|  | Name of button | Function |
| --- | --- | --- |
| 1 | Direction | Use it when you want to move to the desired direction. |
| 2 | Stepping mode | Use the direction buttons to produce the specified displacement during the stepping motion. |
| 3 | Coordinate of robot | Display the robot head position. |
| 4 | Set position | The set position coordinate is saved as XYZU. |
| 5 | Save | Save all data on the current interface. |
| 6 | Move | Move to the saved set position. |

Step11Re-Insert

Select whether to perform re-insert in case of defectivecomponents insertion.

Step12Immediate re- Insert

Stop the current inserting and give priority to re-inserting.

Continue the inserting after the re-inserting, and perform the picking in case of no any other component.

Step13 Angle of Inserting

Enter the angle of inserting based on the "component photographing angle".

In case of negative angle value, the robot will rotate clockwise. In case of positive angle value, the robot will rotate counterclockwise.

Entered angle value ranges - 360 degrees to 360 degrees.

The camera’s X axis is equal to the horizontal direction of the track.

For example: The visual file of component is completed when the length of the component is parallel to the X axis of the camera. At this time, the component insertion angle reference is 0 degrees, parallel to the horizontal direction of the track.

Step14Layer

The order of the insertion can be specified. The layer with the small number is prioritized (in terms of component picking and inserting). The initial value is "1".

The optimized order of the insertion is determined among different layers and also on the same layer. (At the time of production, when the component is used up, if the production of the layer with smaller number has not been finished, the production of the next layer will not be started)

Step15 Trial Insertion

With the Single-component insertion mode, the compensation value for the current component insertion should be adjusted.

Insertion position: Open the "Inserting Position" setting interface, and set the component insertion point and insertion transition point. (it is generally set by trial insertion function)

Step16Insertion Position

Introduction to function of insertion avoidance

X axis: The direction perpendicular to the track is defined as the X axis.

Y axis: The direction parallel to the track is defined as the Y axis.

There may be the component causing interference next to the insertion position of other components, so it is necessary for the robot to drop down for a certain height at the side, perform the horizontal movement to the position above the insertion point and finally to drop down to insert into the component. At this time, the user can set the distance according to the actual situation;

For example: interference during the vertical insertion

demonstrative application of avoidance

Step17Recycling Position

Only used in demonstration mode after the completion of all component insertion on the PCB. The robot will pick up the PCB and components and put them in the recycling position (when the recycling position is not set, the default recycling position is the discharging position).

4.6Use / Non-use of Production components

The settings of use / non-use of production components is introduced as follows.

Use component list

Use list：

Display the currently used components.

Non-use list：

Display the skipped components (spare components).

Add the components to the use list：

Click to select the name of component in the [Non-use List], and click the [Up] button (button of up arrow), finally adding the components to[Use List].

Remove the components from the use list：

Click to select the name of component in the [Use List], and click the [Down] button (button of down arrow), finally removing the components from [Use List].

Error operation: Click to select the component in the [Use List], and click [Up] button. It is invalid. Or click to select the component in the [Non-use list], and click the [Down] button. It is invalid, too.

4.7 Sequence Settings

The route settings interface will be described as follows.

Click the button of [Route Settings], and the interface of route settings will pop up. Click the button of “up” and “down”, and change the order of the chuck No..

4.8Trial Insertion

The interface of trial insertion will be introduced as follows.

Click the button of [Trial Insertion], and the interface of trial insertion will pop up. Follow the steps below.

click the button of [Reset]

Wait for confirmation of the component picking → photographing position

click the button of [Start up]

click the button of [Clamp the PCB]

place the PCB

check the running speed of robot

NO

Adjust the visual file of components

waiting for the track to tightly clamp PCB

place the PCB on the track

wait for completion of reset

confirm the photographing is “OK”

YES

Click the button of [start up] after confirming the photographing is “OK”

Wait for confirmation of Mark point photographing → position above insertion point

NO

Pop-up of interface of compensation for insertion

confirm the insertion result is “OK”

YES

Click the button of [start up] after confirming the photographing is “OK”

click the button of [Move the PCB out]

finish the trial insertion

After the insertion position is set, click the “Save” button on the “Insertion Data” interface to save the settings, and the settings of current component data is completed. Use this method to set all the data of all the components to be produced.

4.9Other Functions

4.9.1

Figure 3-3-1

4.9.2 Automatic Nine-point Calibration

In the "Settings" → "System Settings" → "Robot Settings" → "Nine Point Calibration" interface, set the reference photographing point in the upper left corner of the camera, set the posture position of each clamping gripper, and then click the button of "Start Nine Point Calibration", the robot will automatically perform photographing to realize the function of nine-point calibration.

5 Description of Visual System

5.1 Overview

The vision system, as a new robot plug-in platform for the SmartAITMSR series，It mainly plays the role of providing component pin positioning information and PCB positioning information for the robot.As a part of the entire plug-in system, whether you are familiar with the correct use of this vision system is related to the stability and reliability of the entire plug-in system, so it is important to understand its use in depth. Let's walk into this vision system together.

5.1.1 Main interface

As the main interface of the system, it mainly provides functions such as displaying the initialization results of the vision system, image processing results, visual files, setting whether to save pictures, and manual

Figure1-1

Tool bar

Open image: Select a picture from your hard drive and load it in

Single step acquisition: Get a picture from the camera

Continuous acquisition: Continuous acquisition from the camera

Process Image: Process picture and display results

File manage: Open the file management interface

Save image as: Save the current picture to another path

select file: Choose an already created file

image fit window: Make picture adaptive window

default light setting: Open the default acquisition

Menu bar

Connect Camera: Connect all the cameras

Connect Light Controller: Connect the Light Controller

Refresh file: Reload files again

SystemSetting-> Save image setting: Open the picture save settings interface

Figure 1-2

There are four modes: " Not Save ", "Save OK", "Save Fail", "Save All"。 The function of saving pictures is mainly to save pictures with abnormal production, which is used for problem analysis and modification of files after production.

SystemSetting->opotion: Open system parameter setting interface

Figure 1-3

Camare config: Configure camera serial number

Image save path: Specify the path to save pictures during production, default or custom

5.1.2 File production interface

The file creation interface is where each file is created and modified. Since the interface and process of plug-in file production and Mark point file production are similar, the manual takes plug-in file production as an example to explain，As shown in Figure 1-4。

Figure 1-4

Before making a file, you need to get a qualified photo. You need to set the light source and camera before taking a picture. Click the "Settings" item at the top of the interface and select the "Light settings" item to open the light source camera setting interface. (As shown as figure1-5)，In this interface, you can set the light source channel, brightness, camera gain, and exposure time. At the same time, in conjunction with the movement of the manipulator, you can get a picture with the best lighting effect for file creation.

Figure 1-5 Photo brightness settings

File Name: Display the name of the current file, which can be renamed through the menu file-> rename file

First positioning:

Global positioning is used to locate the approximate position of the entire component. The corresponding positioning tools are shape model match, gray model match, right angle positioning, fit circle positioning, fit rectangle positioning, which will be introduced one by one below.

Second positioning:

Second positioning is pin positioning. You can do two-level positioning. For example, to locate the position of a pin, first add a positioning tool to find the rough position. Generally, we use shape model match, gray model match, blob positioning, Right angle positioning , and then we uses the second tool for precise positioning. The usual tools are circle positioning, rectangle positioning, right angle positioning

As shown in firgure1-6, Each pin uses 2-level positioning, and in figure1-7, Each Pin of figure uses 1-level positioning.

figure1-6

figure 1-7

Add one-time positioning and secondary positioning, first select the corresponding level point, then right-click the mouse to pop up the menu bar, select the corresponding tool, as shown in Figure 1-8-1, Then we add another level 1 tool, as shown in the following four pictures

Figure 1-8-1

Figure 1-8-2

figure 1-8-3

figure 1-8-4

Two tools have been added. Next, you need to set the tools. Double-click the corresponding node to enter the setting interface. The next setting of the tool settings will be explained in detail.

The effect after completion is as follows

Figure 1-8-5

Click the save button to save。

If we find that the accuracy of the primary positioning is not enough, then we need to add secondary positioning, that is, add a level 2 tool

Figure 1-8-6

Figure 1-8-7

Figure 1-8-8

Figure 1-8-9

Figure 1-8-10

After execution, it is found that the picture shows that the processing result is NG. This is because the save has not been performed. Click the save button to pop up the following dialog box.

Figure 1-8-11

Only save tool information: Only modify individual tools, for example, modify the search range of the fit circle. When saving, you must select this default mode.

Save all information: After the new product is created, the first save is performed, or the tool structure of the file is changed, for example, three level 1 tools become four level 1 tools, and the gray model match is deleted and replaced with a gray model match. Situation, you must choose this save mode

After saving and executing, and then performing image processing, the result is normal, as shown below

Figure 1-8-12

After saving, we default the pins of this picture to be standard pins. Next, we need to set the pin offset tolerance in pixels.

Figure 1-8-13

File selection and deletion

Open the tool button open file to open the file selection / delete interface. The interface will display the previously created file. If you want to open a file, double-click it. If you want to remove a file, select the file and click the delete button Just, as shown in Figure 1-9

Figure 1-9

5.2 Processing tools

The previous chapter introduced the entire process of archive production. I believe that I also have a general understanding of the archive production process. This chapter will introduce the several positioning tools mentioned above in detail.

5.2.1 Shape model match

Shape template matching is a contour-based positioning method, which requires that the positioned product has a clearer contour

The tool setting interface is shown below

Figure 2-1

Search range teach:

Click the search range teach button, draw the search range of Pin to be located on the picture, right click after finishing

Add contour:

Click the add contoura button, select the area to extract contours, right-click to end, Figure 2-2 is the extracted contour

Figure 2-2

Dele contour:

Figure 2-3

Figure 2-4

Sometimes the extracted contours are not available for every product. They are interference contours and need to be masked. Click the dele contour button to draw the area to be masked on the image and right-click, As shown in Figures 2-3 and 2-4

Output region teach:

Sometimes the center of the extracted contour is not the real center of the Pin foot. We need our center positioning. Click the output region teach button to draw the center of the Pin foot. Right-click to end, As shown in

Figure 2-5

Param setting:

Angle: Match allowed angular offset

Score: Minimum similarity for template matching, 0 to 100

Save: Save these two parameters to the corresponding Model, The model number is specified in the drop-down list in model lib edit

Model lib edit:

Add to lib: After editing the Model, click the add to lib button to add it to the template library. After the addition is complete, the Model number is displayed in the drop-down list.

from the model library.

Cover model: After creating the template, replace the template specified by the drop-down list

View model: View the template specified by the drop-down list

Remove model: Remove the model specified by the drop-down list

5.2.2Right angle positioning

Right positioning is to search for two intersecting edges and output the intersection point, The interface is shown in the figure 2-6 below.

Figure 2-6

Search range teach:

Draw the edges to be extracted, there are horizontal edges and vertical edges, As shown below, draw a line at the edge that needs to be extracted

Look for :

first: First edge along search direction, Edge search direction is the angle at which we draw the search line plus 90 degrees. For example, if we draw the search line at 90 degrees from left to right, then the search direction is 90 degrees from top to bottom.

best:Best edge along search direction

last: Last edge along search direction

edge type: Polarity of the edges

search length: Search length, as shown below, the shorter side of the green box

edge strength: Minimal edge contrast

Figure 2-7

5.2.3Gray model match

Similar to the operation of the contour template, except that the gray template is based on the correlation of the brightness distribution

Figure 2-8

As shown in the figure above, the template making process is similar to the outline template. First, search the area, then obtain a template, and finally select an output area.

5.2.4 Blob positioning

Blob Analysis is to analyze the connected domains of the same pixels in the image. The connected domains are called Blobs. Blob analysis can provide machine vision applications with the number, location, shape, and orientation of the spots in the image, as well as the topology between related spots. Blob localization (also called blob matching) is obviously applying blob analysis to locate. This tool involves a lot of technical terms, and it is more complicated to understand and use than other tools. Because many professional terms are involved, it is necessary to explain the meaning of individual terms before introducing the tool.

Threshold value：Threshold segmentation is a region-based image segmentation technique. The principle is to divide image pixel points into several categories by setting different feature thresholds. This is the so-called feature threshold.

erosion：The two operations of dilation and erosion are the basis of morphological processing, and many morphological algorithms are based on these two operations. Corrosion is to eliminate the boundary points of the object and make the target smaller, which can eliminate the noise points smaller than the structural elements

diliation：The expansion is to merge all the background points that are in contact with the object into the object to make the target larger and supplement the target.

circularity：It is expressed by the radial offset of the actual contour from the ideal circle, that is, the difference between the maximum radius and the minimum radius relative to the same circle center. You can easily understand the similarity between the feature area and the circle.

rectangularity：Popularly understand the similarity between feature area and rectangle.

Figure 2-9

Interface introduction：

The tool is designed with many parameters. The first version of "Segmentation" mainly plays the role of segmenting the area. The second version of "First Screening" and the third version of "Second Screening" mainly depend on the shape of the feature area , Size, position and other features to filter the segmented graphics. The final step is to restore the shape. The area to be extracted is “circle circle”, “inner circle”, “circle rectangle”, and “rotatable rectangle” to get the area closest to the stitch. One thing to note here is that the second screening can be optional or not. It is mainly based on the analysis of the specific situation. If the first screening can stabilize the SAR to the desired characteristic area, it is not necessary to perform the second screening.

5.2.5 fit circle positioning

However, most of the component pin ends and commonly used Marks are round, so after the initial positioning of the pins by other tools, the shape of the pins is fitted by using a fitting circle method to obtain a more suitable shape for the pins. Round. Can get us more accurate pin positioning, so the fitting circle positioning tool came into being.

Figure 2-10

The principle of fitting a circle can be understood as: after the approximate position of the circular area is initially determined, the transition points are searched for the edges of the circular area, and a certain number of transition points are obtained to fit a standard circle.

Circle teach：Draw a circular contour that fits as closely as possible to determine the search area.

direction：Direction is fixed, search from inside to outside

transition：Search polarity, there are three modes: "all edges", "from dark to light", "from light to dark"。

Look for：Search order selection, there are "first edge", "best edge", "last edge".

minium：Minimal edge change intensity.

kernelsize、projectionwidth：These two parameters are used in combination, which can be understood as the greedy degree of search. The larger the search is, the more accurate it is, and the longer it takes.

Seach gab：How many pixels each search for a point, which is used to fit the circle.

Seach range：The search contours drawn draw the outer and inner ranges respectively.

score：Minimum matching score of the fitted circle

图2-11 拟合圆结果图

5.2.6 fit rectangle positioning

Fitting a rectangle is similar to fitting a circle, so no further explanation is provided here.

5.3 Fast model

For products with too many pins and good lighting effect at the same time, we can use the quick template tool to quickly create files

Figure 3-1

Step 1, Click range teach and draw the area where Pin is, as shown in figure 3-1.

Step 2, Adjust the segmentation threshold, and the minimum and maximum size of the target, so that all the pins you want to extract are displayed. As shown in figure 3-2.

Figure 3-2

Step 3, Select the first-level positioning tool, contour matching or gray model match, and whether to start the second-level positioning tool to fit a circle or a rectangle

Step 4, After selecting various parameters, click the add button to display the template area and search area of each pin, You can then adjust the size of the search area with width and height，By clicking the buttons apply selected and apply all, change the search range of all pins, or specify the search range of pins

Step 5, For unwanted pins or interference areas that come in during extraction, you can click the remove model button to delete them. The number of each area is displayed on the image control. In the leftmost list, select the desired number. Click delete

Finally, close the interface and return to the file management interface, you can see the generated file information, as shown in Figure 3-4

Figure 3-3

Figure 3-4

6 Starting Process

Starting up and shutdown shall be carried out in strict accordance with the guidance in the operation picture. The basic operation process is as shown in the following picture:

Check before operation

Turn on the power switch

Open the software

Designated operator

Confirm production model and components

Machine work

Main interface (reset)

Turn off the power of the machine

Close the software

Turn off the PC

Fig. 4-1 Starting and Shutdown Operational Flow Chart

6.1  Check before Operation

The following items shall be confirmed before the equipment is plugged in :

Attention

Please carry out spot check before the equipment is powered on

■Table-look-up for spot check before operation

| Component under Spot Check | Key Points of Spot Check |
| --- | --- |
| Mechanical Arm | Appearance check to see whether the joints are in good status and there is foreign matters in the actuation range |
| Emergency Exit | whether the exit is damaged and can be closed well or not |
| Transfer Track | Whether there is foreign matter in the transfer track |
| Feeder | the feeder is fixed firmly to the feeder track without and there is no foreign matter at the feeder mouth |
| Gripping gripper | the gripping gripper is with no damage, milky coating, in good resilience and no loose contact |
| component Thrower | Whether the thrower is placed in the designated position and there is no other element in the thrower |

6.2 Starting Steps

The machine is started up according to the following steps

Step1 Rotate the power to on status

Rotate the main switch right under the inserter operation platform to on status to plug in

Step2 [Software startup screen]

Software initialization

Step3 Equipment returning to the home position

The software home screen is displayed after the software is loaded, and all the parts of the equipment initiate movement to home position after the reset button is pressed.

Step4 Press the [login] button

Open the login interface

Step5 Select user and enter password

Switch user user at the pull-down menu of the username

Enter the password corresponding to the username and click [log-in] button, if the password is wrong, a dialogue box would pop out indicating wrong password and if it is correct, the login window is closed automatically, and username level and permission are unlocked. (username permission can be set by the operations of setting-user setting-post management).

7  Finish Production and Turn off Power

The way to finish substrate production is introduced in the following:

Step1Interrupt operation of the machine

There are three ways to interrupt operation of the machine

- Emergency stop button (on the operation panel;)

It is only used in emergency.

- stop button (on the operation panel)

Operation of the machine is interrupted if the stop button is pressed, and operation can be resumed if the [startup]button is pressed.

3.	cycle stop (software production screen)

After the insertion for current substrates is completed, the mechanical arm stops taking component.

The machine stops running after transferring all the substrates with complete insertion out of the track. At this time, the machine continues to run if the [startup] button is pressed.

Scram button in the operation panel

Home

Stop

Start

Emergency stop

Auto/Manual

Reset

Reset

Power

Attention

In the automatic operation process, do not press the scram button if there is no emergency or incoming emergency.

Step2Equipment reset

Press the [Home] button to return back to the status of preparation of substrate production.

Step3Exit the software

Click the [close] button at the right upper corner of the software screen and click [confirmation] button when the dialogue box saying exiting the software is displayed.

Step4Turn off the industrial personal computer

Click the[start] menu button (in the computer desktop) and click the [shutdown] button after the [start] menu displays.

Step5Rotate the main power switch of the machine to OFF status

After the display turns into blank screen, rotate the power switch at the right bottom of the inserter towards left to OFF status.

Attention

Forced shutdown of the power in disregard of the startup and shutdown operation steps would cause data loss, damage of procedure and machine.